Experiment 3

Warm-up exercise

3.1       Flashing of only LEDs

#include "c8051F340.h"

void DelayMs(unsigned int Ms);

#define  LED  P4

void main()

{

P4MDOUT = 0xFF; 	  /* All P4 pins configured as Output	*/

while(1)

{

LED=0X00;              /* LEDs ON*/

DelayMs(100);

LED=0xFF;           /* LEDs OFF	*/

DelayMs(100)

}

}

void DelayMs(unsigned int Ms)

{

unsigned int n;

unsigned int i;

for (n=0; n < Ms; n++)

{

for (i=0; i < 65; i++);

}

}

--------------------------------------------------------------------

3.2: Write an embedded C program to flash the LEDs connected to Port P4 continuously at an interval of 3mSec. Use Timer 0 in Mode 1 for generating the delay.

#include “c8051F340.h”

void Delay();

int main()

{

P4MDOUT = 0xFF;      // Configure Port4 as output

TMOD = 0x01;	          // Select Timer 0 in Mode 1

CKCON = 0x04; 	      // Select System Clock for Timer 0

while (1)

{

P4 = ~P4;	        // Compliment Port4 for flashing

Delay();     // Delay of 3ms between ON and OFF LED

}

return 0;

}

void Delay()

{

TH0 = 0x73; 	      //Load High Byte of Count in TH0

TL0 = 0x52;          //Load Low Byte of Count in TL0

TR0 = 1;	          //Start the Timer

while (TF0 ==0 );    //Wait for overflow

TF0 = 0;	         //Clear overflow flag

TR0 = 0;	         //Stop the Timer

}

--------------------------------------------------------------------------------------------------------------------------------------

3.3 Interfacing of LED, Buzzer, Relay and Switch with C8051F340

#include "c8051F340.h"

#define  LED1  0x01

sbit KEY1 = P1^0;

sbit KEY2 = P1^1;

sbit RELAY = P1^4;

sbit BUZZER = P3^3;

void main()

{

int key , i;

XBR1    = 0x40; 	  /* Enable Crossbar												*/

P1MDIN |= 0x0F;		  /* ALL P1 pins configured as Inputs								*/

P1MDOUT |= 0x30;	  /* ALL P1 pins configured as Inputs								*/

P4MDOUT = 0xFF; 	  /* All P4 pins configured as Output								*/

P3MDOUT = 0x08;

P1		= 0x3F;		  /* All P1 pins are pull-up										*/

P4      = 0xFF; 	  /* All P4 pins are high											*/

P3  =  0x00;

while(1)

{

if(KEY1 == 0)

{ key=1;

if(key ==1)

{

while(1)

{

P4 = 0x00;

RELAY = 0;

BUZZER   = 1;

if(KEY2 == 0)

{

key=0;

break;

}

}

}

}

if(KEY2 == 0)

{ key=1;

if(key ==1)

{

while(1)

{

P4 = 0xFF;

RELAY = 1;

BUZZER   = 0;

if(KEY1 == 0)

{

key=0;

break;

}}

}

}

}

}

*********************************************************************************

Expt.4 Interfacing of LCD with C8051F340

4.1 Display your name.

#include "c8051F340.h"

void DelayMs(unsigned int Ms);

void Write_Command_Lcd(unsigned char Command);

void Write_Data_Lcd(unsigned char Character);

sbit LCD_RS = P1^5;

sbit LCD_RW = P1^6;

sbit LCD_EN = P1^7;

void main()

{

XBR1    = 0x40; 	  /* Enable Crossbar*/

P2MDOUT = 0xFF;

P1MDOUT = 0xE0;

Write_Command_Lcd(0x01);

DelayMs(50);

Write_Command_Lcd(0x38);

DelayMs(50);

Write_Command_Lcd(0X0C);

DelayMs(50);

Write_Command_Lcd(0X80);

DelayMs(50);

Write_Data_Lcd('W');

DelayMs(50);

Write_Data_Lcd('P');

DelayMs(50);

Write_Data_Lcd('U');

DelayMs(50);

while (1);

}

void DelayMs(unsigned int Ms)

{

unsigned int n;

unsigned int i;

for (n=0; n < Ms; n++)

{

for (i=0; i < 65; i++);

}

}

void Write_Command_Lcd(unsigned char Command)

{

LCD_RS = 0;

LCD_RW = 0;

P2 = Command;

LCD_EN = 1; // EN pin high->low

DelayMs(15);

LCD_EN = 0;

}

void Write_Data_Lcd(unsigned char Character)

{

LCD_RS = 1;

LCD_RW = 0;

P2 = Character;

LCD_EN = 1; // EN pin high->low

DelayMs(15);

LCD_EN = 0;

}

4.2:  Rolling Display

#include<c8051f340.h>

void delayms(unsigned int ms);

void writecommandlcd(unsigned char command);

void writedatalcd(unsigned char character);

sbit LCD_RS=P1^5;

sbit LCD_RW=P1^6;

sbit LCD_EN=P1^7;

void main()

{

XBR1=0x40;

P2MDOUT=0xFF;

P1MDOUT=0xE0;

writecommandlcd(0x01);

delayms(50);

writecommandlcd(0x38);

delayms(50);

writecommandlcd(0x0C);

delayms(50);

writecommandlcd(0x80);

delayms(50);

writedatalcd('M');

delayms(50);

writedatalcd('A');

delayms(50);

writedatalcd('S');

delayms(50)

writedatalcd('S');

delayms(50);

while(1)

{

writecommandlcd(0x1C);

delayms(75);

}

}

void delayms(unsigned int ms)

{

unsigned int n;

unsigned int i;

for(n=0;n<ms;n++)

{

for (i=0; i<65; i++)

{

}

}

}

void writecommandlcd(unsigned char command)

{

LCD_RS=0;

LCD_RW=0;

P2=command;

LCD_EN=1;

delayms(15);

LCD_EN=0;

}

void writedatalcd(unsigned char character)

{

LCD_RS=1;

LCD_RW=0;

P2=character;

LCD_EN=1;

delayms(15);

LCD_EN=0;

}

******************************************************************************

Expt. 5 Interfacing DAC with C8051F340.

5.1 For sine wave generation:

#include "c8051F340.h"

int i;

unsigned char x[16]={127,176,218,245,255,245,218,176,128,79,37,10,0,10,37,79};

int main(void)

{

P4MDOUT = 0xFF;

while(1)

{

for(i=0;i<16;i++)

{

P4=x[i];

}

}

}

5.2 Triangular wave generation

#include "c8051F340.h"

int i,j;

void main()

{

P4MDOUT = 0xFF; 	  /* All P4 pins configured as Output	*/

while(1)

{

for (i=0; i <=0xFE; i++)

{

P4 = i;

}

for (j=0xFF; j>= 0; j--)

{

P4 = j;

}

}

}

5.3 :  Sawtooth wave generation:

#include "c8051F340.h"

int i;

void main()

{

P4MDOUT = 0xFF; 	  /* All P4 pins configured as Output					*/

while(1)

{

for (i=0; i <=0xFE; i++)

{

P4 = i;

}

}

}

5.4: Square wave generation:

#include "c8051F340.h"

void DelayMs(unsigned int Ms);

void main()

{

*/

P4MDOUT = 0xFF; 	  /* All P4 pins configured as Output					*/

while(1)

{

P4 = 0x00;

DelayMs(10);

P4 = 0xFF;

DelayMs(10);

}

}

void DelayMs(unsigned int Ms)

{

unsigned int n;

unsigned int i;

for (n=0; n < Ms; n++)

{

for (i=0; i < 165; i++);

}

}

********************************************************************************

Expt. 6: Interfacing ADC with C8051F340.

ADC

//Pin 2.5 as Analog input

#include <c8051F340.h>

#define SYSCLK 12000000

void delay(unsigned int ms_Count) {

unsigned int i,j;

for(i=0;i<ms_Count;i++)

{

for(j=0;j<2500;j++);

}

}

void main ()

{

OSCICN = 0X83;             //System Clock 12 MHz

XBR1 = 0X40;                  // Enable crossbar

P4MDOUT = 0XFF;       // P4 LEDs connected make o/p

P2MDIN = 0XDF;               // P2.5  analog Input

P2SKIP = 0X20;            // Skip P2.5

REF0CN = 0X08;         // Set VDD as reference voltage for  ADC

AMX0P = 0X04;          // P2.5 as ANIP+

AMX0N = 0X1F;        //Connect ANIP- to GND for single ended ADC

ADC0CF = (((SYSCLK/3000000)-1)<<3);

AD0EN =1; 	  // Enable ADC0

ADC0CN = 0X90;    // Give SOC

AD0BUSY=1; 	  // Give SOC

while(AD0BUSY == 1);  //ADC0CN^AD0BUSY==1; Monitor EOC

delay(3000);

{

// Wait for  EOC

}

P4 =~ADC0L;         // Display 10-bit A/D value on LED

delay(3000);   //CommonAnode so ~ LED; LED ON=1, OFF=0

P4 =~ADC0H;

delay(3000);

while(1);

}

***************************************************************

Expt. 7: Interfacing the DC motor and control its speed using PWM with C8051F340.

PCA - DC Motor/PWM

#include <c8051f340.h>

#define SYSCLK       12000000    // Internal oscillator frequency in Hz

void main (void)

{

OSCICN = 0x83; // Set internal oscillator to run at its maximum freq

CLKSEL = 0x00;   // Select the internal osc. as the SYSCLK source

PCA0MD = 0x00;     // Disable watchdog timer

XBR0    = 0x00;

XBR1 = 0x41; / Route CEX0 to P0.0, Enable crossbar and weak pull-up

P1MDOUT = 0x10;   // Set CEX0 (P0.0) to push-pull

P0SKIP = 0xFF;

P1SKIP = 0x0F;

while (1)

{

// Configure PCA time base; overflow interrupt disabled

PCA0CN = 0x00;                      // Stop counter; clear all flags

PCA0MD = 0x08;                      // Use SYSCLK as time base

PCA0CPM0 = 0x42;                    // Module 0 = 8-bit PWM mode

// Configure initial PWM duty cycle = 50%

PCA0CPH0 = 256 - (256 * 0.5);

// Start PCA counter

CR = 1;

}

}

Expt. No. 8: Implement UART with C8051F340

8.1 //Transmission

#include "c8051F340.h"

#define		SYSCLK		12000000

#define		BR_UART0		9600

void main()

{

char ch[]={"MIT"};

int i;

SCON0= 0x00;

OSCICN |= 0x83;       /*  Configure internal oscillator for 						its maximum frequency 12MHz */

XBR0	= 0x01;

XBR1    = 0x40; 	  	/* Enable Crossbar												*/

P0MDOUT = 0x10; 	  	/* P0 pin configured as Output	*/

TH1 = -(SYSCLK/BR_UART0/2/4); /*we know that 									(SYSCLK/BR_UART0/2/256 < 4) so 							the prescale is 	by 4 */

/* T1M = 0; SCA1:0 = 01 */

/* negative sign gives   								compliment so it enters value 							256-#VALUE */

CKCON &= ~0x0B;

CKCON |= 0x01;

TL1 = TH1;		/* Init Timer1 */

TMOD &= ~0xf0;	/* TMOD: timer 1 in 8-bit autoreload */

TMOD |= 0x20;

TR1 = 1; 		/* START Timer1 */

for (i=0;ch[i]!='\0';i++)

{

SBUF0 = ch[i];

while(TI0 != 1);

TI0 = 0;

}

while(1);

}

// 8.2. Reception

#include "c8051F340.h"

#define                SYSCLK                  12000000

#define                BR_UART0                          9600

void main()

{

//char ch[]={"MIT"};

int i;

OSCICN |= 0x83;       /*  Configure internal oscillator for               its maximum frequency 12MHz*/

SCON0= 0x10;

XBR0 = 0x01;

XBR1    = 0x40;                   /* Enable Crossbar*/

P0MDIN = 0x20;                                /* P0 pin configured as Output*/

P4MDOUT=0xFF;

TH1 = -(SYSCLK/BR_UART0/2/4); /*we know that (SYSCLK/BR_UART0/2/256 < 4) so the prescale is by 4 */                                                                /* T1M = 0; SCA1:0 = 01 */

/* negative sign gives   compliment so it enters value 256-#VALUE */

CKCON &= ~0x0B;

CKCON = 0x01;

SCON0=0x10;

TL1 = TH1;                           /* Init Timer1 */

TMOD &= ~0xf0;               /* TMOD: timer 1 in 8-bit autoreload */

TMOD |= 0x20;

TR1 = 1;                                /* START Timer1 */

while(RI0 == 0)

{

P4 = ~SBUF0;

RI0 = 0;

}

}

********************************************************************

Expt. No. 9: Interfacing Stepper Motor with C8051F340.

9.1 Rotation in clockwise direction:

#include "c8051F340.h"

void DelayMs(unsigned int Ms);

void main()

{

P4MDOUT = 0xFF;     /* All P4 pins configured as Output*/

while(1)

{

P4 = 0x0A;

DelayMs(50);

P4 = 0x88;

DelayMs(50);

P4 = 0xA0;

DelayMs(50);

P4 = 0x22;

DelayMs(50);

}

}

void DelayMs(unsigned int Ms)

{

unsigned int n;

unsigned int i;

for (n=0; n < Ms; n++)

{

for (i=0; i < 65; i++);

}

}

// 9.2 Rotation in Anti-clockwise direction

#include "c8051F340.h"

void DelayMs(unsigned int Ms);

void main()

{

P4MDOUT = 0xFF;     /* All P4 pins configured as Output*/

while(1)

{

P4 = 0x22;

DelayMs(20);

P4 = 0xA0;

DelayMs(20);

P4 = 0x88;

DelayMs(20);

P4 = 0x0A;

DelayMs(20);

}

}

void DelayMs(unsigned int Ms)

{

unsigned int n;

unsigned int i;

for (n=0; n < Ms; n++)

{

for (i=0; i < 65; i++);

}

}

Expt 10 : Interfacing EEPROM using SPI with C8051F340

Only attach code( No Performance)

#include "c8051F340.h"

#include "types.h"

#define SPI_CLOCK    3000000            // Maximum SPI clock

#define  SYSCLK      12000000     /* SYSCLK in Hz (OSCICN is configured as 12MHz) */

/* the internal oscillator has a tolerance of +/- 2% */

#define EEP_WRITE  0x02;  /* Write bit of IR     */

#define EEP_READ   0x03;  /* Read bit of IR      */

#define EEP_WREN   0x06;  /* Write Enable Latch  */

#define EEP_WRDI   0x04;  /* Write Disable Latch */

#define EEP_RDSR   0x05;  /* Read Status Reg     */

#define EEP_WRSR   0x01;  /* Write Status Reg    */

void DelayMs(unsigned int Ms);

void Write_Command_Lcd(unsigned char Command);

void Write_Data_Lcd(unsigned char Character);

schar Write_Data_Spi(schar *Data, char Length);

schar Read_Data_Spi(schar *Data, char Length);

char Spi_Eeprom_Write_Byte(uint16 eep_address, char byte_data);

char Spi_Eeprom_Read_Byte(uint16 eep_address);

sbit LCD_RS = P1^5;

sbit LCD_RW = P1^6;

sbit LCD_EN = P1^7;

void main()

{

char a[] = {"Write Successful"};

char a1[] = {"Write Failed"};

int i;

char status;

char read;

OSCICN |= 0x83;

RSTSRC  = 0x04;

XBR0      = 0x03;   // Enable the SPI and Uart0 on the XBAR

XBR1      = 0x40;

//P0MDOUT   = 0x1D;   //Tx output

//XBR1    = 0x40; 	  /* Enable Crossbar*/

P2MDOUT = 0xFF;

P1MDOUT = 0xE0;

SPI0CFG |= 0x40;                   // Enable the SPI as a Master

SPI0CN  = 0x0D;                    // 4-wire Single Master, SPI enabled

//SPI clock frequency equation from the datasheet

SPI0CKR = (SYSCLK/(2*SPI_CLOCK))-1;

//ESPI0 = 1;                       // Enable SPI interrupts

SPI0CFG &=~0x30; //mode0: CKPHA = '0', CKPOL = '0'

//SPI0CFG |= 0x30; //mode3

Write_Command_Lcd(0x01);

DelayMs(50);

Write_Command_Lcd(0x38);

DelayMs(50);

Write_Command_Lcd(0X0C);

DelayMs(50);

Write_Command_Lcd(0X80);

DelayMs(50);

status = Spi_Eeprom_Write_Byte(0x05, 'A');       /* Write Byte to Address   */

if(status)

{

for (i=0; a[i]!='\0'; i++)

Write_Data_Lcd (a[i]);

}

else

{

for (i=0; a[i]!='\0'; i++)

Write_Data_Lcd (a1[i]);

}

Write_Command_Lcd(0XC0);

DelayMs(50);

read = Spi_Eeprom_Read_Byte(0x05);

Write_Data_Lcd (read);

DelayMs(50);

while (1);

}

void DelayMs(unsigned int Ms)

{

unsigned int n;

unsigned int i;

for (n=0; n < Ms; n++)

{

for (i=0; i < 65; i++);

}

}

void Write_Command_Lcd(unsigned char Command)

{

LCD_RS = 0;

LCD_RW = 0;

P2 = Command;

LCD_EN = 1; // EN pin high->low

DelayMs(15);

LCD_EN = 0;

}

void Write_Data_Lcd(unsigned char Character)

{

LCD_RS = 1;

LCD_RW = 0;

P2 = Character;

LCD_EN = 1; // EN pin high->low

DelayMs(15);

LCD_EN = 0;

}

char Spi_Eeprom_Write_Byte(uint16 eep_address, char byte_data)

{

char SpiData[4],WriteData,WriteStatus =0;

SpiData[0] = EEP_WRITE;                  /* WRITE IR 8bit msb       */

SpiData[1] = (char)(eep_address>>8);    // 1st byte extract

SpiData[2] = (char) eep_address;        // 2nd byte extract

SpiData[3] = byte_data;                  /* byte data lsb           */

WriteData = EEP_WREN;

NSSMD0 = 0;

Write_Data_Spi(&WriteData,1);            /* Write Enable Latch      */

NSSMD0 = 1;

NSSMD0 = 0;                              /* CS low active           */

WriteStatus = Write_Data_Spi(SpiData,4); /* Pass value              */

if(WriteStatus)

{

NSSMD0 = 1;                            /* CS high inactive        */

return(1);

}

else

return(0);

}

schar Write_Data_Spi(schar *Data, char Length)

{

while(Length)

{

if(!(SPI0CN&0x40))

{

SPI0DAT = *Data++;

while(!SPIF);

SPIF = 0;

Length--;

}

else

return(0);

}

return(1);

}

schar Read_Data_Spi(schar *Data, char Length)

{

while(Length)

{

SPI0DAT = 0xFF;

while(!SPIF);

SPIF = 0;

*Data++ = SPI0DAT;

Length--;

}

return(1);

}

char Spi_Eeprom_Read_Byte(uint16 eep_address)

{

char SpiData[3],eep_data,ReadStatus =0;

SpiData[0] = EEP_READ;                 /* READ IR 8bit msb        */

SpiData[1] = (char)(eep_address>>8);  // 1st byte extract

SpiData[2] = (char)eep_address;       // 2nd byte extract

NSSMD0 = 0;

Write_Data_Spi(SpiData,3);             /* Pass address            */

ReadStatus = Read_Data_Spi(&eep_data,1); /* Store byte in eep_data  */

if(ReadStatus)

{

NSSMD0 = 1;

return(eep_data);                    /* Return value            */

}

else

return(0);

}